#

File: onkyo.info,  Node: Top,  Prev: (oldtech.info),  Next: (orion.info),  Up: (remotes.info)

******************************************************************************

ONKYO

******************************************************************************

* Menu:

* Technical info::

* Amp::

* CD::

#

File: onkyo.info,  Node: Technical info,  Prev: Top,  Next: Amp,  Up: Top

******************************************************************************

TECHNICAL INFO

******************************************************************************

Code length:            32 bits

Carrier:                39.2kHz

The code is only transmitted once, and after that a stop code is transmitted.

+--------------------------------+

Header:      |                                |

+                                +----------------+..

16T                       8T

8.8ms                     4.4ms

+--+

1 is coded:  |  |

..+  +------+..

T   3T

+--+

0 is coded:  |  |

..+  +--+..

T  T

+--+

|  |

After the last bit a ( ..+  +  ) is transmitted to terminate the last bit.

T

+--------------------------------+        +--+

Stop code:    |                                |        |  |

+                                +--------+  +

16T                  4T     T

8.8ms                2.2ms

T = 550us

Space between transmissions: 95000us between the headers of the codes.

To use the code (not with the remote package):

GROUP * 10000h + (Code * 100h) + (Code XOR FFh)

For more information please read the documentation.

#

File: onkyo.info,  Node: Amp,  Prev: Technical info,  Next: Tuner,  Up: Top

******************************************************************************

AMP

******************************************************************************

Device: Onkyo amplifier (name: A-8780)

GROUP:  #4BB6h

Code    Function

----    --------

#20     power on/off

#40     volume +

#C0     volume -

#A0     muting

#10     tape 1

#E0     tape 2

#D0     tuner

#90     cd

#30     dat

#60     aux 1

#88     aux 2

#50     phono

#

File: onkyo.info,  Node: Tuner,  Prev: Amp,  Next: Amp,  Up: Top

******************************************************************************

TECHNICAL INFO

******************************************************************************

The Onkyo RI system allows to control devices like a tuner or a cd

(or tape 1&2) via cable link between the amplifier and the device.

This certainly works only on other Onkyo components with the IR-logo,

connected twith the amplifier.

tuner (via link from amplifier)

Code    Function

----    --------

#00     channel +

#80     channel -

#08     scan

#52     class

#A2     antenna

#62     fm

#E2     am

cd (via link from amplifier)

Code    Function

----    --------

#D8     play

#F8     pause

#38     stop

#78     track <<

#B8     track >>

Device:         Onkyo tuner (name: T-4670)

GroupCode:      #4BA4h

Function                        Code (hex)      Remarks

----------------------------------------------------------------------------

#00     channel +

#80     channel -

#08     scan

#32     class

#D8     shift

#62     rf mode

#E2     if band

#12     hi-blend

#92     mode (mono/stereo)

#A2     antenna

#90     fm

#50     am

#38     1

#88     2

#48     3

#C8     4

#18     5

#98     6

#58     7

#B0     8

#70     9

#F0     10

#0A     11

#8A     12

#4A     13

#CA     14

#2A     15

#AA     16

#6A     17

#EA     18

#1A     19

#9A     20

Device:         Onkyo cd (name: DX-6770)

GroupCode:      #4B34h

Code    Function

----    --------

#04     play

#D8     play

#F8     pause

#38     stop

#84     stop

#D0     open/close

#78     track <<

#B8     track >>

#80     search <<

#00     search >>

#40     level +

#C0     level -

#50     elapse/remain

#62     shuffle

#7A     fader

#90     memory/memory reverse

#A2     intro scan

#60     repeat

#E0     a-b repeat

#32     auto space

#BA     peak search

#B2     time edit

#C2     edit <-

#42     edit ->

#10     clear

#F0     +10

#70     0

#08     1

#88     2

#48     3

#C8     4

#18     5

#98     6

#58     7

#30     8

#B0     9

#F2     10

#0A     11

#8A     12

#4A     13

#CA     14

#2A     15

#AA     16

#6A     17

#EA     18

#1A     19

#9A     20